SWMM5 LEGO Builder

Technical Handover Document

Version 1.0 | March 2026

Browser-Based Stormwater Model Builder with Full LEGO Visual Aesthetic

# 1. Executive Summary

SWMM5 LEGO Builder is an interactive, browser-based stormwater management model builder with a full LEGO visual aesthetic. Users paint surfaces, place drainage nodes, draw pipe networks, run hydrologic/hydraulic simulations, and export/import EPA SWMM5 .INP files — all entirely in the browser with zero backend dependencies.

The application features dual simulation engines: a lightweight JavaScript engine for instant visual feedback and the official EPA SWMM5 v5.2 solver compiled to WebAssembly for publication-quality results.

# 2. Technology Stack

| Layer | Technology | Notes |
| --- | --- | --- |
| Framework | React 18 (JSX) | Single-page application |
| Build Tool | Vite 7.3 | Hot module replacement in dev |
| Charts | Recharts 2.15 | LineChart, BarChart, ResponsiveContainer |
| Styling | Inline styles + LegoToolbar.css | LEGO 3D brick aesthetic |
| Fonts | Google Fonts | Fredoka (headings), Nunito (body) |
| WASM Engine | EPA SWMM5 v5.2.4 | Emscripten 2.0.10 compilation |
| Persistence | localStorage | Auto-save + 5 named slots |
| Hosting | Replit | pnpm monorepo workspace |

# 3. Project Structure

Total codebase: ~5,000 lines across 15 source files.

## 3.1 File Inventory

| File | Lines | Purpose |
| --- | --- | --- |
| SWMM5LegoBuilder.jsx | 2,201 | Main React component — grid, UI, simulation state, all panels |
| LegoToolbar.css | 542 | LEGO 3D brick styling (studs, depth borders, press animations) |
| elements.js | 37 | Element definitions (EL object), grid config, categories |
| storms.js | 87 | 49 design storm profiles from 6 continents |
| demos.js | 365 | 13 pre-built demo model builder functions |
| hydraulics.js | 276 | JS simulation engine (SCS-CN + Manning's equation) |
| swmmWasm.js | 80 | EPA SWMM5 WASM wrapper (Emscripten loader, virtual FS I/O) |
| parseRpt.js | 266 | .RPT text report parser (summary tables) |
| parseOut.js | 220 | .OUT binary output parser (time-series extraction) |
| exportInp.js | 62 | .INP file generator for EPA SWMM5 |
| importInp.js | 280 | .INP file importer with coordinate mapping |
| exportCsv.js | 49 | CSV time-series exporter |
| exportHtmlReport.js | 365 | Standalone HTML graph report generator |
| validation.js | 134 | Model validation + CFL checks + auto-fix |
| persistence.js | 37 | localStorage save/load management |

## 3.2 WASM Binaries

| File | Size | Source |
| --- | --- | --- |
| public/swmm/js.wasm | 452 KB | EPA SWMM5 v5.2.4 compiled with Emscripten |
| public/swmm/js.js | 109 KB | Emscripten JavaScript glue code |

# 4. Grid System & Elements

The application uses a 2D grid (default 20x20, configurable up to 50x50) where each cell represents a 100 ft x 100 ft area. Users paint cells with three categories of elements:

## 4.1 Surface Elements (Subcatchment Generators)

| Element | Curve Number | % Impervious | Manning's n | Color |
| --- | --- | --- | --- | --- |
| Grass | 61 | 0 | 0.15 | #70C442 |
| Roof | 98 | 100 | 0.012 | #D01012 |
| Road | 98 | 100 | 0.013 | #6C6E68 |
| Driveway | 98 | 100 | 0.016 | #9CA3AF |
| Sidewalk | 98 | 100 | 0.014 | #E4CD9E |
| LID Pond | 45 | 0 | 0.25 | #38BDF8 |
| Permeable Paving | 72 | 50 | 0.018 | #A78BFA |
| Green Roof | 55 | 0 | 0.20 | #22C55E |
| Rain Barrel | 61 | 0 | 0.15 | #60A5FA |
| Swale | 55 | 0 | 0.25 | #15803D |

## 4.2 Node Elements (Drainage Junctions)

| Element | Max Depth (ft) | Description |
| --- | --- | --- |
| Manhole | 6 | Standard junction node |
| Inlet | 4 | Surface inlet/catch basin |
| Outfall | 0 | Discharge point (required in every model) |
| Storage | 10 | Detention/retention basin |
| Divider | 6 | Flow splitting node |

## 4.3 Link Elements (Conveyance)

| Element | Diameter (ft) | Manning's n | Description |
| --- | --- | --- | --- |
| Pipe | 1.5 | 0.013 | Circular conduit |
| Channel | 3.0 | 0.025 | Open channel |
| Pump | — | — | Mechanical pump |
| Orifice | 1.0 | — | Flow restriction |
| Weir | 3.0 | — | Overflow structure |

# 5. Dual Simulation Engines

The application features two independent simulation engines with fundamentally different capabilities:

## 5.1 Quick Sim (JavaScript Engine)

Purpose: Instant visual feedback for iterating on network design.

- SCS Curve Number method for infiltration losses

- Manning's equation for overland flow and pipe flow

- 15-second routing timestep with kinematic wave routing

- Explicit forward time-stepping with real-time grid animation

- Cannot model: backwater effects, surcharge, reverse flow, storage routing

Key Functions: runSWMM5(), buildModel(), manningPipe(), manningOverland(), cnInfiltration()

## 5.2 EPA SWMM5 WASM Engine

Purpose: Publication-quality results identical to desktop EPA SWMM5.

- Dynamic Wave (DynWave) routing — full Saint-Venant equations

- Accounts for backwater effects, surcharge, reverse flow, storage routing

- Produces standard .RPT text report + .OUT binary time-series

- WASM binary: EPA SWMM5 v5.2.4 compiled from C to WebAssembly

## 5.3 Why Two Engines?

The dual-engine approach is pedagogically valuable: users iterate quickly with Quick Sim to understand their network, then run the full EPA engine for definitive results. The difference between the two teaches about model uncertainty and the importance of proper hydraulic routing.

# 6. Binary Output Parser (parseOut.js)

The EPA SWMM5 WASM engine produces a binary .out file containing full time-series data for every object at every reporting timestep. The parser reads this file to extract:

## 6.1 Extracted Variables

| Object Type | Variables |
| --- | --- |
| Subcatchments (8) | rainfall, snowDepth, evapLoss, infilLoss, runoff, gwOutflow, gwElev, soilMoisture |
| Nodes (6) | depth, head, volume, lateralInflow, totalInflow, flooding |
| Links (5) | flow, depth, velocity, volume, capacity |
| System (14) | airTemp, rainfall, snowDepth, evapInfilLoss, runoff, dryWeatherInflow, gwInflow, rdiiInflow, directInflow, totalLatInflow, flooding, outfallFlow, storageVolume, actualEvap |

## 6.2 Binary File Format

| Section | Contents |
| --- | --- |
| Header | Magic (0x1ED0EE1A), Version, Flow Units, Object Counts |
| ID Names | Subcatchment/Node/Link/Pollutant string IDs |
| Properties | Object property codes + float values (area, invert, maxDepth, etc.) |
| Variables | Reporting variable codes for each object type |
| Metadata | Start date (double), Report step (seconds) |
| Results | For each period: date (double) + subcatch/node/link/system float arrays |
| Footer | Section offsets, Period count, Error code, Magic number |

# 7. Design Storms (49 Profiles)

Storm profiles are organized by region, each defined as a fractional cumulative distribution with total depth:

| Region | Storm Profiles |
| --- | --- |
| US Standards | SCS Type I, IA, II, III; NOAA Atlas 14; Huff Quartiles 1-4; USACE Standard Project; PMP (HMR 51/52) |
| US State/Local | Florida FDOT Zones 1-5; Texas TxDOT; Denver UDFCD |
| Europe | UK FSR/FEH; Chicago Storm; French Desbordes; German DWA-A 531, Euler Type I/II; Dutch STOWA; Italian Mediterranean |
| Asia-Pacific | Japan JMA, AMeDAS, Baiu, Typhoon; China GB 50014, Pearl River Delta; India IMD Monsoon; Singapore PUB; Australian ARR |
| Other | South African Huff; Canadian CDA, MTO; Block (Uniform); Double Peak; Yen & Chow Triangular |

# 8. Demo Scenarios (13 Models)

| Demo | Description |
| --- | --- |
| Residential | Suburban neighborhood with houses, lawns, trunk sewer |
| Parking Lot | Impervious commercial lot with LID treatment |
| Green Infra | Bioretention focus, minimal impervious cover |
| Highway | Corridor with shoulders, medians, roadside drainage |
| Mixed-Use | Downtown block with buildings and LID plaza |
| School Campus | Athletic fields, buildings, dual outfalls |
| Industrial | Large warehouse roofs, heavy-duty drainage |
| Hillside | Steep terrain, cascading manholes, erosion control |
| Hospital | Emergency access, redundant drainage |
| Dual Outfall | Split drainage basins, separate discharge points |
| Stadium (Full) | Large-scale roof drainage, concourse collection |
| Stadium Simple | Simplified stadium variant |
| Minimal Test | Minimal 3-cell test model |

# 9. Export & Import Capabilities

| Format | Direction | Description |
| --- | --- | --- |
| SWMM5 .INP | Export | Standard EPA SWMM5 input file, compatible with desktop SWMM5 |
| SWMM5 .INP | Import | Parse standard .inp files, map coordinates to grid, place elements |
| SWMM5 .RPT | Export | Raw text report download from EPA WASM engine |
| CSV | Export | Time-series data for system/subcatchment/node/pipe tabs |
| HTML Report | Export | Self-contained HTML with Canvas-rendered charts, LEGO themed, printable |

# 10. Model Validation

## 10.1 Error Checks (Block Simulation)

- Empty model (no elements placed)

- No outfall node present

- No link elements (pipes/channels) present

- Disconnected pipes (link cells not connecting two nodes)

## 10.2 Warning Checks

- No junction nodes (only outfall)

- Flat pipes (zero slope from row-based elevation)

- CFL/Courant number exceeds 1.0 (numerical instability risk)

- Very short or very long pipe runs

## 10.3 Auto-Fix

autoFix(grid) attempts automatic corrections: adds missing outfall, adds missing inlet, logs all changes made.

# 11. User Interface

## 11.1 Desktop Layout (>768px)

- Left Panel (210px): Element palette, surface/node/link categories, paint/erase toggle

- Center: Interactive grid with LEGO baseplate aesthetic

- Right Sidebar (280px): Storm selector, simulation controls, result charts, inspector, documentation

## 11.2 Mobile Layout (<768px)

- Single-column stacked layout with dynamic cell sizing

- Touch support: single-finger paint/drag, long-press (500ms) for property editor

- Hidden panels: DUAL ENGINES, SHORTCUTS

## 11.3 LEGO Visual Theme

| Color | Hex | Usage |
| --- | --- | --- |
| Red | #D01012 | Errors, outfalls, EPA results, roof elements |
| Green | #70C442 / #4B9F4A | Grass, success states, subcatchment charts |
| Blue | #006DB7 / #5A93DB | Pipes, system charts, panel headers |
| Yellow | #F2C717 | Warnings, highlights, tutorial overlay |
| Orange | #FE8A18 | CFL warnings, node results |
| Gray | #6C6E68 | Roads, neutral elements, close buttons |
| Navy | #1B2A34 | Text, dark backgrounds, body |
| Off-white | #F4F4F4 | Panel backgrounds |

# 12. Keyboard Shortcuts

| Key | Action |
| --- | --- |
| Ctrl+Z | Undo last change |
| Space | Toggle paint/erase mode |
| Del | Activate erase mode |
| R | Run Quick Sim |
| Esc | Close open panels |
| 1-9 | Select surface element (Grass, Roof, Road, etc.) |
| Shift+1-5 | Select node element (Manhole, Inlet, Outfall, etc.) |
| Q / W / E | Select Pipe / Channel / Pump |

# 13. Data Persistence

| localStorage Key | Contents |
| --- | --- |
| swmm5-lego-autosave | Auto-saved on every change: grid, gridSize, stormIdx, cellProps |
| swmm5-lego-saves | Array of up to 5 named save slots with name, data, and date |

# 14. Known Limitations

- Quick Sim: Kinematic wave only — no backwater, surcharge, reverse flow, or storage routing

- Grid-based model: Limited to regular grid layouts; cannot represent arbitrary network topologies

- Single rain gage: All subcatchments share the same rainfall time series

- No water quality: Pollutant transport not implemented

- Cell-based elevation: Elevation derived from grid row position (top = high, bottom = low)

- localStorage only: No cloud save, no multi-user collaboration

- Browser memory: Very large models (50x50 with long simulations) may consume significant memory

# 15. Build & Deployment

Development: pnpm --filter @workspace/swmm5-lego run dev

Production Build: pnpm --filter @workspace/swmm5-lego run build

Type Check: pnpm --filter @workspace/swmm5-lego run typecheck

The application is deployed on Replit and requires no backend server. All simulation runs entirely in the browser.

End of Handover Document